III. SYSTEM OVERVIEW AND PROBLEM FORMULATION

We introduce a magnetically actuated, non-contact rehabilitation system designed to reduce the physical risks associated with rigid robotic interaction while preserving an engaging motor-training task. This section first describes the hardware architecture and then formulates the control problem as a reinforcement learning (RL) task.

A. Hardware Platform

The platform improves safety by physically separating the robotic actuator from the patient. As shown in Fig. X, the system consists of three layers. The actuation layer places a UR10 robotic arm beneath the workspace, with a permanent magnet mounted on the end-effector to drive the microrobot above. The interaction layer is a 5-mm acrylic sheet that supports the unpowered magnetic microrobot and serves as a barrier between the arm and the patient. The perception layer uses an overhead RGB camera connected to a host computer, which also communicates with the UR10 over Ethernet. At each control cycle, the computer acquires a camera frame, estimates the positions of the microrobot and the patient's hand, and sends target pose commands to the arm.

This physical separation is central to the safety design. The acrylic interface reduces the risk of blunt impact, excessive traction, and crushing injuries that may occur in direct-contact rehabilitation devices. The hardware is also simple and reproducible: it requires a collaborative robot, a camera, and a permanent magnet, without wearable components, force sensors, or custom actuation modules. Because the microrobot is unpowered and driven passively by the magnetic field, it does not require onboard electronics or batteries. This simplicity facilitates setup and maintenance, and it provides an additional safety margin during the iterative development of data-driven controllers.

B. Problem Formulation

We formulate the rehabilitation task as a pursuit-evasion game on a flat workspace. The patient sits in front of the acrylic surface and uses one hand to chase the magnetic microrobot, which is controlled by the robot arm below. The goal is to bring the hand within a small contact radius of the microrobot. Unlike static reaching exercises, the microrobot actively evades the patient's hand, requiring continuous visuomotor tracking, trajectory prediction, and real-time motor planning. Such goal-directed tracking can impose sustained cognitive and physical demands beyond point-to-point reaching and is relevant to motor recovery [X]. To maximize therapeutic benefit, task difficulty must adapt to the patient's current ability. We formalize this requirement using the Zone of Proximal Development (ZPD), which describes tasks that remain challenging but achievable with support [X]. In our setting, the ZPD corresponds to a target distance range between the hand and the microrobot. The robot should neither escape completely nor yield passively; instead, it should keep the interaction within this range so that the patient remains engaged without becoming fatigued or frustrated. This objective is challenging because patient movement speed, strategy, delay, and motor noise can vary over time.

Conventional controllers are not well suited to this setting. Methods based on artificial potential fields or rule-based impedance control usually rely on hand-crafted responses to instantaneous state feedback. They therefore have limited ability to anticipate higher-level patient strategies, such as interception, or to adapt to heterogeneous movement patterns. Their performance may degrade further under tremor, delayed response, or bradykinesia because they do not explicitly distinguish purposeful movement from involuntary perturbations. RL provides a natural framework for learning such adaptive behavior through repeated interaction. In our formulation, the robot learns an evasion policy that is rewarded for keeping the hand-microrobot distance within the therapeutic range and penalized for unsafe or uninformative behavior, such as escaping too far or being caught too easily. By conditioning the policy on temporal observations, the controller can infer patient intent from movement history rather than reacting only to instantaneous position error. The design choices in Section IV--domain randomization, temporal encoding, and league training--address these sources of patient variability.

IV. METHODOLOGY

This section describes the training framework. We first introduce Cognitive-Motor Decoupled Domain Randomization (CMD-DR), which separates patient movement strategy from motor execution limitations. We then present a dual-stream encoder with an auxiliary future-dynamics head for extracting motion information from noisy temporal observations. Finally, we describe a league-training procedure designed to improve robustness against a growing pool of hand behaviors.

A. Cognitive-Motor Decoupled Domain Randomization

A generalizable robot policy must account for variation in both patient strategy and motor execution. CMD-DR addresses these two factors separately. The first layer randomizes the hand strategy by sampling a controller from an opponent pool at the beginning of each episode. The pool contains scripted pursuit controllers and trained RL controllers that can intercept or anticipate the microrobot. The selected controller produces a desired hand displacement at each timestep. The second layer transforms this desired displacement through motor and sensing constraints. Specifically, the raw displacement is passed through a first-order low-pass filter to model muscle inertia:

x_t = alpha x_{t-1} + (1 - alpha) u_t,   alpha in (0, 1).     (1)

where u_t denotes the raw desired displacement and x_t denotes the filtered displacement. Acceleration clipping is then applied to limit abrupt changes in motion:

Delta x_t = clip(x_t - x_{t-1}, -a_max, a_max).     (2)

Gaussian observation noise simulates position-estimation errors from the vision system. A sampled temporal delay models delayed motor response by executing a displacement generated several frames earlier. Table I lists the parameter values and sampled ranges for these constraints. By separating intended movement from motor execution, CMD-DR exposes the robot policy to diverse patient-like behaviors and reduces dependence on any single simulated hand model.

| Parameter | Symbol | Range |
| --- | --- | --- |
| Muscle inertia | alpha | 0.7 |
| Maximum acceleration | a_max | 0.15 |
| Gaussian observation noise | sigma | U(0.01, 0.08) |
| Neural delay | delta | 0-3 frames (0-375 ms) |

B. Dual-Stream Encoder with Future Dynamics Head

A single frame is often insufficient for inferring patient intent. Observed hand motion may reflect sensing noise, motor delay, tremor-like perturbations, or slow response, rather than deliberate pursuit. We therefore use a dual-stream encoder with an auxiliary prediction task, as shown in Fig. X.

The encoder separates geometric context from motion history. At each timestep, the 44-dimensional observation vector is divided into a 12-dimensional scalar vector and a 32-dimensional temporal buffer:

o_t = [s_t ; h_{t-T:t}].     (3)

The scalar vector is defined as:

s_t = [p_R; p_H; d(R,H); b_N, b_S, b_E, b_W; stride; a_{t-1}].     (4)

where p_R and p_H denote the two-dimensional positions of the microrobot and the hand, respectively; d(R,H) is their Euclidean distance; b_N, b_S, b_E, and b_W are the signed distances to the workspace boundaries; stride is the current movement step size; and a_{t-1} is the previous robot action. These variables describe the current geometric state of the interaction, including relative position, boundary distance, and recent action information.

The temporal buffer h_{t-T:t} contains the T = 16 most recent relative hand displacement vectors, forming a 32-dimensional sequence. This buffer captures velocity, acceleration, and repeated movement patterns that are unavailable from the current position alone. The scalar vector is processed by a Multi-Layer Perceptron (MLP) to extract geometric information. In parallel, the temporal buffer is processed by a Gated Recurrent Unit (GRU) sequence encoder. Because the GRU operates on relative displacements rather than absolute positions, it emphasizes movement direction, speed, acceleration, and oscillation. The two stream outputs are concatenated and passed through a fusion MLP to produce a shared representation for policy and value prediction.

The ZPD-based reward provides a control signal, but it does not directly supervise how the encoder should represent patient motion. Learning this representation only through policy reward can therefore be slow and unstable. To provide a denser signal, we attach a future-dynamics head to the fused representation. This head predicts the patient's relative displacement over an eight-step horizon and estimates near-catch risk. Because future displacements are already available from the rollout, the auxiliary targets require no additional annotation. The trajectory loss is defined as:

L_traj = E[||D_hat_{t+1:t+H} - D_{t+1:t+H}||_2^2],   H = 8.     (5)

where D_hat_{t+1:t+H} is the predicted future displacement sequence and D_{t+1:t+H} is the observed future displacement sequence. The full training objective combines the PPO loss with the auxiliary trajectory and risk-prediction losses:

L_total = L_PPO + lambda_traj L_traj + lambda_risk L_risk.     (6)

where lambda_traj and lambda_risk control the auxiliary loss weights. Because the future-dynamics head shares the encoder with the policy, its gradients also update the spatial and temporal streams. This encourages the encoder to capture patient inertia, delay, and interaction dynamics, enabling the policy to anticipate near-future hand motion rather than reacting only to the current hand-microrobot distance.

C. League Training

Training both policies simultaneously is a straightforward baseline, but it creates a non-stationary learning problem. The return of each agent depends on the policy of the other agent. When both policies are updated together, each update changes the environment faced by the other policy, violating the stationarity assumption of Proximal Policy Optimization (PPO). In practice, this can lead to cyclic adaptation and forgetting: the robot exploits the current hand policy, the hand adapts, and the robot loses strategies that were useful against earlier hands. We avoid this issue by freezing one side of the interaction while training the other. During robot training, the hand policies are fixed and treated as part of the environment. During hand training, the robot policy is fixed.

The objective also differs from purely competitive games. In rehabilitation, the robot should not simply learn to defeat the hand. It should maintain an appropriate level of challenge for patients with different movement abilities. A single hand policy cannot represent this range of behavior, so we maintain an opponent pool P containing hand policies from different training stages. This pool provides a diverse set of pursuit behaviors for robot training.

The full procedure is formalized in Algorithm 1. Each iteration has two phases. In the robot phase, the opponent pool P is fixed, and the robot policy is trained with PPO against hand policies sampled from the pool. In the hand phase, a new hand policy is initialized from the previous hand policy and further trained against the current fixed robot policy. The trained hand policy is then added to P, increasing the diversity of hand behaviors used in the next robot training phase.

Algorithm 1: Iterative League Training

Initialize: opponent pool P <- {pi_H^script}, robot policy pi_R^(0)

for n = 1, 2, ..., N do

// Robot phase (pool frozen)

Train pi_R^(n) via PPO against opponents sampled

from P using prioritized distribution P(i)

// Hand phase (robot frozen)

Warm-start pi_H^(n) from pi_H^(n-1), fine-tune

against frozen pi_R^(n)

P <- P union {pi_H^(n)}

end for

Uniform sampling from the opponent pool is inefficient. Weak hand policies provide little learning signal because the robot can maintain the desired distance too easily, whereas very strong hand policies can terminate episodes before useful feedback is obtained. The most informative opponents are challenging but not impossible for the current robot. We therefore track the robot's average normalized episode length against each hand policy and sample opponents using a length-prioritized distribution:

P(i) propto f(w_i),  f(w) = 1 / (1 + exp(-k * (eta - |w - 0.5|))).     (7)

where w_i denotes the normalized episode length against opponent i, k controls the sharpness of the distribution, and eta defines the tolerance band around the target value of 0.5. This sampling rule gives higher probability to hand policies that produce intermediate episode lengths. In this way, training effort is automatically concentrated on the most informative opponents without requiring a manually specified curriculum.

One possible risk of league training is that the robot may learn to exploit artificial patterns in the simulated hand policies. Such behavior would reduce the relevance of the learned policy for real rehabilitation. To reduce this risk, a fixed probability p_s is always assigned to the heuristic scripted hand policy. This ensures that the robot remains exposed to simple, noisy pursuit behavior, which is closer to the behavior expected from real patients.

VI. EXPERIMENTS AND RESULTS

The experiments evaluate the proposed framework from complementary perspectives rather than following the methodology section mechanically. We first define the common simulation setup, training protocols, and evaluation metrics. The main evaluation then examines whether league training improves robustness across learned hand behaviors and whether this robustness transfers across scripted and learned-agent hand controllers. We next analyze the role of temporal interaction history and the auxiliary future-dynamics head through a network ablation. The section closes with the real-world deployment pipeline that connects the simulation policy to the UR10 platform.

A. Experimental Setup

All simulation experiments use the pursuit-evasion rehabilitation environment described in Section III. The robot controls the magnetic microrobot, and the hand is controlled either by a scripted pursuit controller or by a learned hand policy. The therapeutic objective is defined by a ZPD distance band of 3.5-5.5 workspace units. A step is counted as in-zone when the hand-microrobot distance lies inside this band.

The primary evaluation metric is Time-In-Zone (TIS), the fraction of the episode horizon during which the interaction remains inside the ZPD band. We also report ZPD coverage, episode length, catch rate, and empirical robustness across the learned hand pool. These metrics capture different aspects of the same rehabilitation objective: TIS measures sustained therapeutic interaction over the full horizon, ZPD coverage measures the quality of the realized portion of an episode, episode length captures survival, and catch rate reflects overly easy interactions in which the microrobot is caught too quickly.

We compare three representative robot training protocols. The scripted-only baseline is trained against the stochastic scripted pursuit controller. The single-agent baseline is trained against one learned hand policy. The league policy is trained through iterative opponent-pool expansion, where robot generations are trained against a mixture of scripted and learned hand behaviors. For the network ablation, we compare an MLP policy, a GRU policy with temporal interaction history, and a GRU+Aux policy with the auxiliary future-dynamics head.

B. League Training and Robustness Evaluation

This evaluation tests whether opponent-pool training reduces the brittle specialization that can occur when the robot is optimized against a single hand model. The league policy is trained for ten generations while the hand pool is expanded with learned hand policies. During robot training, the opponent identity is not given as an input; the robot must infer the current hand behavior from the spatial state and recent displacement history. This design makes the evaluation closer to rehabilitation use, where patient capability is observed through movement rather than provided as a label.

The league analysis in Fig. 1 is computed by freezing each robot generation and evaluating it against the learned hand pool under the same ZPD band. For each robot-hand pairing, we measure the fraction of time spent inside the ZPD and then summarize performance in three ways: the mean trend over the pool, the lower-tail or worst-case trend, and an empirical matrix of pairwise outcomes. The mean curve indicates overall adaptation, whereas the worst-case curve is used to detect whether some hand policies remain failure cases. The matrix provides the most detailed view because it shows which robot generations are robust across easy and difficult virtual patients.

The resulting pattern is not only an increase in average performance. Later league generations also improve the lower tail, meaning that the robot becomes less dependent on favorable hand behaviors. This distinction is important for rehabilitation: a policy that works well for typical patients but collapses for difficult movement patterns would not provide reliable personalized training. The sorted matrix supports the same interpretation, showing that later generations maintain useful ZPD interaction across a broader portion of the learned-hand pool.

The scripted-hand curve in Fig. 1 is used as a consistency check rather than the main robustness test. The scripted controller follows a noisy pursuit rule and therefore probes whether league training preserves basic tracking behavior. The learned-hand matrix is more diagnostic because learned opponents can expose strategy-dependent weaknesses. Taken together, the figure indicates that opponent accumulation improves robustness without relying on an explicit opponent label.

Fig. 1. League-training overview for the no-opponent-ID ZPD 3.5-5.5 simulation. The final robot generation improves mean and worst-case Time-In-Zone against the learned-hand pool, while the sorted empirical matrix visualizes robustness across increasingly difficult virtual patients.

We further test whether this robustness transfers across hand-controller types. For this comparison, the final scripted-only, single-agent, and league-trained robot policies are each evaluated against two hand mechanisms: the stochastic scripted pursuit controller and a learned hand agent. The table entries report TIS, ZPD coverage, episode length, and catch rate in that order. Mouse-controlled human-in-the-loop testing is kept separate because it introduces human reaction time and voluntary strategy rather than another automated controller.

TABLE II: POLICY COMPARISON ON SCRIPTED AND LEARNED-AGENT HANDS

| Robot policy | Scripted hand | Agent hand |
| --- | --- | --- |
| Scripted-only | 0.31 / 0.54 / 52 / 60% | 0.19 / 0.54 / 31 / 90% |
| Single-agent | 0.11 / 0.41 / 25 / 95% | 0.49 / 0.60 / 70 / 38% |
| League | 0.38 / 0.55 / 67 / 43% | 0.48 / 0.65 / 64 / 48% |

Table II shows complementary failure modes for the two non-league baselines. The scripted-only robot remains competitive on the scripted controller but is caught frequently by the learned hand agent, indicating that rule-based pursuit during training does not cover learned strategic behavior. The single-agent robot shows the reverse tendency: it performs well against the learned hand but degrades on the scripted controller. The league-trained robot does not dominate every metric in every column, but it avoids the severe collapse observed in the baselines and provides the most balanced behavior across both automated test conditions. This balance is the relevant outcome for an adaptive rehabilitation controller, whose goal is not to overfit to one hand model but to maintain therapeutic interaction as patient behavior changes.

C. Network Ablation and Auxiliary Dynamics Analysis

The ablation study turns from opponent diversity to policy representation. The key question is whether the robot can regulate difficulty from the current geometric state alone, or whether it needs recent patient-motion history. The MLP baseline observes the current geometric state but does not explicitly encode recent interaction history. The GRU policy processes the 16-frame relative-displacement buffer, allowing it to infer hand velocity, response delay, and pursuit tendency. The GRU+Aux policy further adds the future-dynamics prediction head described in Section IV-B.

The upper panels of Fig. 2 show that the main performance gain appears when temporal interaction history is introduced. Both recurrent policies learn longer and more rewarding interactions than the MLP baseline. This suggests that the robot benefits from observing how the hand has been moving, not only where it is at the current instant. In this setting, the GRU-only and GRU+Aux policies follow similar aggregate learning trends, indicating that temporal sequence modeling is the dominant contributor to control performance.

The lower panels of Fig. 2 provide a complementary interpretation of the auxiliary head. Rather than treating the auxiliary task as a separate source of reward improvement, we use it to inspect whether the temporal encoder has learned predictive structure in hand motion. The predicted trajectories capture short-horizon movement direction and diverge gradually at longer horizons, consistent with the uncertainty of multi-step prediction in an interactive task. Thus, the auxiliary head mainly serves to encourage and expose a temporal representation of patient dynamics.

Fig. 2. GRU ablation and auxiliary future-motion prediction. Panels (a) and (b) show smoothed training reward and episode length for MLP, GRU, and GRU+Aux policies trained against the mixed learned-hand and scripted-hand pool. Panels (c) and (d) visualize representative auxiliary future-prediction examples and horizon-dependent trajectory error.

D. Real-World Deployment Pipeline

The final part of the experiment section describes how the simulation policy is connected to the physical UR10 platform. This pipeline is presented as an implementation bridge rather than as a paired sim-to-real benchmark, because real hand kinematics, camera latency, magnetic actuation, and safety constraints are not fully represented in the virtual training environment.

The physical implementation uses the same 44-dimensional observation structure as simulation: robot position from Real-Time Data Exchange (RTDE), hand and microrobot positions from the overhead camera, boundary features, previous action, and the displacement-history buffer. Perception and control run in separate threads to avoid jitter. The camera runs at 10-15 Hz, whereas the robot requires deterministic commands at a fixed control rate. If both processes ran in one thread, a slow You Only Look Once (YOLO) inference step could stall the control loop.

The system therefore uses two threads connected by a single-element message queue. The vision thread captures frames, runs undistortion and detection, and pushes results to the queue. If the queue is full, stale results are replaced. The control thread runs at 20 Hz and attempts a non-blocking read each cycle. If a new frame is available, the hand position is updated; otherwise, the system uses dead reckoning based on a low-pass filtered velocity estimate from recent vision updates. The resulting observation is passed to the PPO policy, and the two-dimensional action is mapped through the inverse homography before being sent to the UR10 through the servoL interface.