III. SYSTEM OVERVIEW AND PROBLEM FORMULATION

To reduce the physical risks of rigid rehabilitation robots and provide an engaging therapeutic environment, we introduce a magnetically-actuated non-contact rehabilitation system. This section describes the hardware architecture, states the control problem, and formulates it as a reinforcement learning task.

A. Hardware Platform

The platform achieves safety by physically separating the robotic actuator from the patient. As shown in Fig. X, the system has three layers. The actuation layer places a UR10 robotic arm beneath the workspace, with a permanent magnet on its end-effector that drives the microrobot above. The interaction layer is a 5 mm acrylic sheet that supports the unpowered magnetic microrobot while acting as a barrier between the arm and the patient. The perception layer uses an overhead RGB camera connected via USB to a host computer, which also communicates with the UR10 over Ethernet. At each control cycle the computer acquires a camera frame, estimates the positions of the microrobot and the patient's hand, and sends target pose commands to the arm.

By separating the robotic actuator from the patient's limb, the acrylic interface substantially reduces the risk of blunt impact, excessive traction, and crushing injuries that may arise in direct-contact rehabilitation devices. In addition to improving safety, the system has a simple and reproducible structure: it requires only a collaborative robot, a camera, and a permanent magnet, without wearable components, force sensors, or custom actuation modules. The microrobot itself is unpowered and is driven passively by the magnetic field, eliminating the need for onboard electronics or batteries. This hardware simplicity facilitates setup, maintenance, and potential deployment across clinical environments. Moreover, the non-contact actuation mechanism provides an additional safety margin for deploying and refining learning-based controllers on the physical platform.

B. Problem Formulation

We formulate the rehabilitation task as a pursuit-evasion game played on a flat workspace. The patient sits in front of the acrylic surface and uses one hand to chase the magnetic microrobot, which moves under the control of the robot arm below. The goal is to catch the microrobot by bringing the hand within a small contact radius. This is not a static reaching exercise: the microrobot actively evades, so the patient must continuously track, predict, and intercept a moving target, which demands sustained attention and real-time motor planning. Clinical evidence shows that this kind of continuous, goal-directed visuomotor tracking engages the central nervous system and promotes neuroplasticity for motor recovery [X]. To maximize therapeutic benefit, task difficulty must adapt to the patient's current ability. This principle is formalized by the Zone of Proximal Development (ZPD), a concept from educational psychology introduced by Vygotsky [X] that describes the range of tasks a learner can perform with guidance but not alone. In our setting, the ZPD translates to a target distance range between the hand and the microrobot: the robot should neither escape completely nor yield passively, but keep the distance within this range so the patient stays challenged without becoming fatigued or frustrated.

Maintaining this therapeutic difficulty range is challenging for conventional control methods. Many classical approaches, such as artificial potential fields or rule-based impedance controllers, rely primarily on hand-crafted rules and instantaneous state feedback. As a result, they may struggle to anticipate higher-level patient strategies, such as trajectory interception, or to adapt to heterogeneous movement patterns across patients. Their performance may further degrade in the presence of neurological motor impairments, including tremor, delayed response, and bradykinesia, because these methods typically lack an explicit mechanism for distinguishing purposeful movement from involuntary perturbations. Reinforcement learning provides a suitable framework for addressing these challenges. In our formulation, the robot learns an evasion policy through repeated interaction with the patient or patient models. The policy is rewarded for maintaining the hand-microrobot distance within the therapeutic range and penalized for unsafe or uninformative behaviors, such as escaping too far or being caught too easily. By conditioning the policy on temporal observations of hand and microrobot motion, the controller can infer patient intent from movement history rather than responding only to instantaneous position error. Through this process, the robot can progressively adapt its behavior to different patient capabilities and maintain an appropriate level of challenge. The following sections describe how the policy is trained to generalize across diverse movement patterns observed in neurological rehabilitation.

IV. METHODOLOGY

The previous section formulated the rehabilitation task and motivated the use of reinforcement learning. This section describes the complete training framework. We first introduce a domain randomization scheme that accounts for both patient movement strategies and motor limitations (Section IV-A). We then present a dual-stream encoder with an auxiliary forward dynamics head to extract motion information from noisy temporal observations (Section IV-B). Finally, we describe a league training procedure that improves the robustness of the robot policy against a growing pool of hand behaviors (Section IV-C).

A. Cognitive-Motor Decoupled Domain Randomization

The pursuit-evasion task is implemented as a Gymnasium environment with two interacting entities. The robot policy controls the magnetic microrobot, while the hand represents the patient and is controlled by a separate policy or scripted controller. A key challenge for real-world deployment is that neurological patients may move in very different ways. If the simulated hand follows only one fixed strategy, the learned robot policy may overfit to that behavior and perform poorly when facing real patients. To address this issue, we use a two-layer domain randomization scheme. The first layer randomizes the hand's movement strategy. At the beginning of each episode, a hand controller is sampled from an opponent pool. This pool includes simple scripted controllers based on fixed pursuit rules as well as trained RL controllers that can intercept and anticipate the microrobot. The selected controller produces a desired hand displacement at each timestep. The second layer modifies this desired displacement according to motor and sensing constraints. The raw displacement is first passed through a first-order low-pass filter to model muscle inertia:

x_t = alpha x_{t-1} + (1 - alpha) u_t,   alpha in (0, 1),

where u_t denotes the raw desired displacement and x_t denotes the filtered displacement. Acceleration clipping is then applied to limit abrupt changes in motion:

Delta x_t = clip(x_t - x_{t-1}, -a_max, a_max).

Gaussian observation noise is further added to simulate position estimation errors from the vision system. To model delayed motor response, a temporal delay is introduced so that the displacement executed at time t corresponds to the intended displacement generated several frames earlier. The delay is sampled at the beginning of each episode. This design separates what the hand attempts to do from how the movement is actually executed. As a result, the robot policy is exposed to a wide range of patient-like behaviors and is encouraged to learn motion patterns that are not tied to a single simulated hand model.

| Parameter | Symbol | Range |
| --- | --- | --- |
| Muscle inertia | alpha | 0.7 |
| Maximum acceleration | a_max | 0.15 |
| Gaussian observation noise | sigma | U(0.01, 0.08) |
| Neural delay | delta | 0-3 frames (0-375 ms) |

B. Dual-Stream Encoder with Future Dynamics Head

The patient's intended movement cannot be directly inferred from a single frame. In addition, the observed hand motion may be affected by sensing noise, motor delay, tremor-like perturbations, and slow response. Therefore, using only instantaneous position inputs may not be sufficient for the policy to distinguish purposeful movement from short-term disturbances. To address this problem, we design a dual-stream encoder with an auxiliary prediction task, as shown in Fig. X.

At each timestep, the 44-dimensional observation vector is divided into a 12-dimensional scalar vector and a 32-dimensional temporal buffer:

o_t = [s_t ; h_{t-T:t}].

The scalar vector is defined as:

s_t = [p_R; p_H; d(R,H); b_N, b_S, b_E, b_W; stride; a_{t-1}],

where p_R and p_H denote the two-dimensional positions of the microrobot and the hand, respectively; d(R,H) is their Euclidean distance; b_N, b_S, b_E, and b_W are the signed distances to the workspace boundaries; stride is the current movement step size; and a_{t-1} is the previous robot action. These variables describe the current geometric state of the interaction, including relative position, boundary distance, and recent action information.

The temporal buffer h_{t-T:t} contains the T = 16 most recent relative hand displacement vectors, forming a 32-dimensional sequence. This buffer captures recent velocity, acceleration, and repeated movement patterns that cannot be obtained from the current position alone. The scalar vector is processed by an MLP to extract the current geometric information. In parallel, the temporal buffer is processed by a GRU sequence encoder. Because the GRU uses relative displacements rather than absolute positions, it focuses on movement direction, speed, acceleration, and oscillation. The outputs of the two streams are then concatenated and passed through a fusion MLP to produce a shared representation for policy and value prediction.

Although the ZPD-based reward provides a continuous training signal, it does not directly teach the encoder how to represent patient motion. Learning this representation only through policy reward can be slow and unstable. To provide a more direct signal, we attach a future-dynamics head to the fused representation. This head predicts the patient's future relative displacement over an eight-step horizon and also estimates near-catch risk. Since future displacements are already contained in the rollout, these targets can be obtained without extra data collection or manual annotation. The auxiliary trajectory loss is defined as:

L_traj = E[||D_hat_{t+1:t+H} - D_{t+1:t+H}||_2^2],   H = 8,

where D_hat_{t+1:t+H} is the predicted future displacement sequence and D_{t+1:t+H} is the observed future displacement sequence. The full training objective combines the PPO loss with the auxiliary trajectory and risk-prediction losses:

L_total = L_PPO + lambda_traj L_traj + lambda_risk L_risk,

where lambda_traj and lambda_risk control the auxiliary loss weights. Because the future-dynamics head shares the encoder with the policy, its gradients also update the spatial and temporal streams. This encourages the encoder to capture patient inertia, delay, and interaction dynamics, helping the robot policy anticipate near-future hand motion instead of reacting only to the current hand-microrobot distance.

C. League Training

A natural baseline is to train the robot and hand policies at the same time. However, simultaneous training in a two-agent game creates a non-stationary learning problem. The return of each agent depends on the policy of the other agent. When both policies are updated together, each update changes the environment faced by the other policy, which conflicts with the stationarity assumption of PPO. In practice, this can lead to cyclic adaptation: the robot learns to exploit the current hand policy, the hand then adapts to the robot, and the robot may forget strategies that were useful against earlier hand behaviors. To avoid this issue, we freeze one side of the interaction while training the other. During robot training, the hand policies are fixed and treated as part of the environment. During hand training, the robot policy is fixed. This alternating procedure provides a more stable learning objective than updating both policies simultaneously.

Our goal is different from purely competitive games. In rehabilitation, the robot should not simply learn to defeat the hand. Instead, it should maintain an appropriate level of challenge for patients with different movement abilities. A single hand policy cannot represent this range of behavior. We therefore maintain an opponent pool P that stores hand policies from different training stages. These hand policies provide a diverse set of pursuit behaviors for training the robot.

As summarized in Algorithm 1, each iteration has two phases. In the robot phase, the opponent pool P is fixed, and the robot policy is trained with PPO against hand policies sampled from the pool. In the hand phase, a new hand policy is initialized from the previous hand policy and further trained against the current fixed robot policy. The trained hand policy is then added to P, increasing the diversity of hand behaviors used in the next robot training phase.

Algorithm 1: Iterative League Training

Initialize: opponent pool P <- {pi_H^script}, robot policy pi_R^(0)

for n = 1, 2, ..., N do

// Robot phase (pool frozen)

Train pi_R^(n) via PPO against opponents sampled

from P using prioritized distribution P(i)

// Hand phase (robot frozen)

Warm-start pi_H^(n) from pi_H^(n-1), fine-tune

against frozen pi_R^(n)

P <- P union {pi_H^(n)}

end for

Uniformly sampling from the opponent pool is inefficient. Weak hand policies provide little learning signal because the robot can maintain the desired distance too easily. Very strong hand policies may lead to mostly failed episodes, which also provide limited useful feedback. The most useful hand policies are those that are challenging but not impossible for the current robot policy. To focus training on these useful opponents, we track the robot's average normalized episode length against each hand policy and sample opponents using a length-prioritized distribution:

P(i) propto f(w_i),  f(w) = 1 / (1 + exp(-k * (eta - |w - 0.5|)))

where w_i denotes the normalized episode length against opponent i, k controls the sharpness of the distribution, and eta defines the tolerance band around the target value of 0.5. This sampling rule gives higher probability to hand policies that produce intermediate episode lengths. In this way, the robot is trained more often against opponents that match its current ability, without manually specifying a difficulty schedule.

One possible risk of league training is that the robot may learn to exploit artificial patterns in the simulated hand policies. Such behavior would reduce the relevance of the learned policy for real rehabilitation. To reduce this risk, a fixed probability p_s is always assigned to the heuristic scripted hand policy. This ensures that the robot remains exposed to simple, noisy pursuit behavior, which is closer to the behavior expected from real patients.

VI. EXPERIMENTS AND RESULTS

The simulation study is organized around the main sources of uncertainty that the controller must handle before physical deployment. We first examine whether iterative league training produces a robot policy that remains effective across a growing set of learned hand behaviors. We then isolate the role of temporal interaction history and auxiliary future-dynamics supervision through a controlled network ablation. Finally, we compare representative robot policies under two automated hand-control mechanisms to assess whether the learned behavior is tied to a single opponent type or transfers across different pursuit strategies.

A. League Training Robustness

The league experiment uses a ZPD band of 3.5-5.5 workspace units and trains ten robot generations without providing an explicit opponent identity. The absence of opponent identity makes the task more representative of rehabilitation, where the controller must infer patient capability from motion history rather than from a predefined label. Each robot generation is evaluated against the learned hand pool and a scripted pursuit controller. Time-in-zone (TIS), defined as the fraction of interaction time spent inside the ZPD band, is used as the main robustness metric, with both mean and worst-case values reported to capture average performance and lower-tail robustness.

Fig. 1 shows that robustness improves progressively as the opponent pool becomes more diverse. The final-generation robot achieves the strongest overall performance against learned hands, and the improvement is visible not only in the mean TIS curve but also in the worst-case trend. This distinction is important for rehabilitation: a policy that performs well only on average may still fail for patients whose movement patterns are difficult or atypical. The league result therefore suggests that opponent accumulation improves the lower tail of performance, rather than merely optimizing for an easy subset of virtual patients.

The sorted empirical matrix provides a complementary view of the same phenomenon. Early robot generations perform well only against a limited portion of the hand pool, whereas later generations maintain useful ZPD interaction across a broader range of opponent difficulty. The scripted-hand evaluation remains useful as a consistency check, but it is less diagnostic than the learned-hand matrix because it does not expose strategy-dependent failures. This motivates the use of league-trained virtual patients as a stress-test distribution for adaptive rehabilitation policies.

Fig. 1. League-training overview for the no-opponent-ID ZPD 3.5-5.5 simulation. The final robot generation improves mean and worst-case time-in-zone against the learned-hand pool, while the sorted empirical matrix visualizes robustness across increasingly difficult virtual patients.

B. GRU Network Ablation

The architecture ablation compares three robot policy encoders under the same mixed-opponent protocol. The MLP baseline observes the current geometric state but does not explicitly encode recent interaction history. The GRU variant adds a temporal sequence encoder over the 16-frame relative-displacement buffer, allowing the policy to infer hand velocity, response delay, and pursuit tendency. The GRU+Aux variant further augments this recurrent representation with the auxiliary future-prediction objective described in Section IV-B. This comparison is designed to distinguish whether performance gains arise primarily from temporal memory or from the additional self-supervised prediction signal.

Fig. 2 summarizes the ablation result. The most prominent change occurs when temporal interaction history is introduced: both recurrent policies learn longer and more rewarding interactions than the MLP baseline. This indicates that the controller benefits from observing how the hand has been moving, not only where the hand is at the current instant. In contrast, the auxiliary variant follows a trajectory close to the GRU-only policy in the aggregate training curves. This suggests that, under the current training setting, temporal sequence modeling is the dominant contributor to control performance, while the auxiliary objective mainly shapes the learned representation.

The auxiliary prediction panels clarify why this component is still useful. Even when the policy-level improvement is modest, the future-dynamics head exposes what the temporal representation has learned about hand motion. The predicted trajectories capture short-horizon movement direction and progressively diverge at longer horizons, which is consistent with the uncertainty of multi-step forecasting in an interactive task. Thus, the auxiliary task should be interpreted less as an independent performance booster in this run and more as a mechanism for encouraging and diagnosing patient-dynamics representation.

Fig. 2. GRU ablation and auxiliary future-motion prediction. Panels (a) and (b) show smoothed training reward and episode length for MLP, GRU, and GRU+Aux policies trained against the mixed learned-hand and scripted-hand pool. Panels (c) and (d) visualize representative auxiliary future-prediction examples and horizon-dependent trajectory error.

C. Policy Comparison on Scripted and Learned-Agent Hands

The final comparison evaluates whether the learned robot behavior transfers across different hand-control mechanisms. The scripted controller represents a simple noisy pursuit strategy, whereas the learned hand agent can exploit interaction patterns acquired through RL training. A robust rehabilitation controller should not be tuned exclusively to either case; it should maintain useful interaction when the hand behavior shifts between rule-based pursuit and learned strategic pursuit.

TABLE II: POLICY COMPARISON ON SCRIPTED AND LEARNED-AGENT HANDS

| Robot policy | Scripted hand | Agent hand |
| --- | --- | --- |
| Scripted-only | 0.31 / 0.54 / 52 / 60% | 0.19 / 0.54 / 31 / 90% |
| Single-agent | 0.11 / 0.41 / 25 / 95% | 0.49 / 0.60 / 70 / 38% |
| League | 0.38 / 0.55 / 67 / 43% | 0.48 / 0.65 / 64 / 48% |

The table reports TIS / ZPD coverage / episode length / catch rate. The scripted-only baseline behaves as expected: it remains relatively effective on the scripted hand but is frequently caught by the learned agent. The single-agent baseline shows the opposite pattern, performing well against the learned hand while degrading on the scripted controller. These complementary failures indicate that training against a single opponent type can produce brittle specialization. The league-trained robot avoids the strongest form of either failure mode and provides the most balanced behavior across the two automated test conditions. Mouse-controlled human-in-the-loop evaluation will be reported after collection.

D. Sim-to-Real Transfer

The physical UR10 deployment is not directly comparable to simulation through a pointwise sim-real gap table, because real human behavior, camera latency, magnetic actuation, and safety constraints are not distribution-matched to virtual hands. We therefore treat sim-to-real validation as a real-world closed-loop feasibility study rather than as a paired simulation benchmark. The implementation uses the same 44-dimensional observation structure as simulation: robot position from RTDE, hand and microrobot positions from the overhead camera, boundary features, previous action, and the displacement-history buffer.

The implementation uses temporal decoupling between perception and control. The camera runs at 10-15 Hz, but the robot needs deterministic commands at a fixed control rate. Running both in one thread would cause jitter: a slow YOLO inference would stall the control loop and freeze the robot. We therefore use two threads connected by a single-element message queue. The vision thread captures frames, runs undistortion and detection, and pushes results to the queue. If the queue is full, the stale result is replaced. The control thread runs at 20 Hz and attempts a non-blocking read each cycle. If a new frame is available, the hand position is updated; otherwise the system falls back to dead reckoning using a low-pass filtered velocity estimate from recent vision updates.

The 44-dimensional observation is assembled from the estimated hand position, the robot position via RTDE, and the displacement-history buffer, then fed to the PPO policy. The 2D action is accumulated as a virtual target in pixel space, rate-limited to prevent large per-step motion, and mapped to world coordinates via the inverse homography. The final pose is sent to the UR10 through the servoL interface with a 0.1 s lookahead time and proportional gain of 400, yielding smooth end-effector tracking.

VII. DISCUSSION

Taken together, the simulation results indicate that robustness emerges from two interacting design choices. The first is exposure to a diverse opponent pool: league training reduces dependence on a single hand model and improves performance across both average and difficult learned opponents. The second is temporal inference: the recurrent encoder allows the robot to condition its behavior on recent hand motion, which is necessary for anticipating pursuit rather than reacting only to distance. The scripted-versus-agent comparison further shows why both aspects are needed. Policies trained against a single opponent type can appear strong under matched evaluation but fail when the hand behavior changes, whereas the league-trained policy provides a more balanced response across test conditions.

The auxiliary future-prediction task should be viewed as a representation-learning component rather than as a guaranteed source of immediate reward improvement. Its value is that it provides dense supervision on patient motion and makes the learned temporal representation inspectable through predicted trajectories. The current results suggest that the auxiliary signal is directionally useful but not yet optimally coupled to the control objective. Future work should therefore study loss weighting, prediction horizon, and curriculum timing to better translate representation quality into policy improvement.

VIII. CONCLUSION

This manuscript presents a simulation-to-real RL framework for adaptive non-contact upper-limb rehabilitation. CMD-DR generates diverse virtual patient behavior by separating intended movement strategy from motor execution constraints. A dual-stream recurrent encoder uses interaction history to infer latent patient dynamics, and an auxiliary future-prediction head supplies dense self-supervised representation learning. Iterative league training improves robustness across learned hand opponents, and the GRU ablation confirms the importance of temporal sequence modeling. The scripted-versus-agent comparison further shows that league training provides more balanced behavior than policies trained against a single opponent type. The next stage is to complete mouse-controlled testing and systematic physical validation on the UR10 magnetic microrobot platform.