Stage Manuscript Draft: Methods, Experiments, and Physical Deployment

Abstract, Introduction, Discussion, Conclusion, and References remain to be finalized.

Stage manuscript draft revised from `methodology_ieee_v2.44_physical_deployment_polish.md`. This version merges fragmented result paragraphs, removes configuration-heavy deployment details from the main prose, simplifies the PFSP sampling formula, and prepares Algorithm 1 for formal Word rendering.

# II. SYSTEM OVERVIEW AND PROBLEM FORMULATION

We introduce a magnetically actuated, non-contact rehabilitation system for active upper-limb motor training. The user interacts with an unpowered magnetic microrobot on a tabletop workspace, while a robotic arm drives the microrobot indirectly through a permanent magnet placed below the surface. This layout preserves an active pursuit task for the user while avoiding rigid physical coupling between the robot and the human body. It places the user in a pursuit role rather than a passive following role, aligning the task with the need for active engagement in motor rehabilitation. We formulate the control problem as a reinforcement learning (RL) task in which the robot regulates interaction difficulty through the motion of the microrobot.

## A. Hardware Platform

The platform separates actuation, interaction, and perception into three physical layers. In the actuation layer, a UR10 robotic arm is placed beneath the workspace, and a permanent magnet is mounted on its end-effector to drive the microrobot above. In the interaction layer, a 5-mm acrylic sheet supports the unpowered microrobot and forms a physical barrier between the robotic arm and the user. In the perception layer, an overhead red-green-blue (RGB) camera sends images to a host computer, which estimates the positions of the microrobot and the hand and transmits target pose commands to the UR10 over Ethernet.

This layout supports both safety and reproducibility. Fig. 1 summarizes the complete simulation-to-real workflow, from policy training in the pursuit-evasion simulator to camera-based observation construction and physical magnetic actuation. The acrylic sheet creates a fixed separation between the actuator and the user-side workspace, so the participant interacts only with the passive microrobot rather than with the robot arm. The microrobot is lightweight and unpowered, and commanded motion can be limited in software while the UR10 remains below the surface with its standard emergency stop available. The hardware also requires only a collaborative robot, a camera, a calibrated planar workspace, and a permanent magnet. It does not rely on wearable components, force sensors, onboard batteries, or custom actuators, which makes the setup easier to reproduce across sessions.

Fig. 1. System framework for simulation-to-real non-contact rehabilitation. The simulation environment defines the pursuit-evasion interaction, the policy network maps spatial and temporal observations to microrobot actions, the image-to-observation module converts camera measurements into policy inputs, and the physical platform executes the learned policy through magnetic actuation.

## B. Problem Formulation

We cast the task as a closed-loop pursuit-evasion interaction on a flat workspace. The participant's hand acts as the pursuer, and the microrobot acts as a moving target driven by the robot arm beneath the acrylic surface. The participant attempts to approach and catch the microrobot, while the robot controls the microrobot motion to keep the task challenging but still reachable. This differs from static reaching because the target does not remain fixed after a movement is planned. The participant must continuously track, anticipate, and correct hand motion, making the task a sustained visuomotor exercise rather than an isolated point-to-point reach [1].

To keep the task challenging but achievable, we define a Zone of Proximal Development (ZPD) as a target distance range between the hand and the microrobot [2]. The robot should neither escape completely nor yield passively. Instead, it should keep the interaction inside this range so that the user remains engaged without the task becoming trivial or unreachable. Hand speed, pursuit strategy, response delay, and motion noise can all change during interaction, making fixed difficulty settings inadequate.

Conventional controllers are limited in this setting because methods such as artificial potential fields or rule-based impedance control usually rely on hand-crafted responses to instantaneous state feedback. These methods struggle to anticipate strategic pursuit behaviors such as interception or to adapt to heterogeneous movement patterns. Their performance can also degrade under delayed response or noisy motion, since purposeful movement and perturbation are not explicitly separated. These limitations motivate an RL formulation in which the controller optimizes a long-horizon ZPD objective under variable hand behavior rather than reacting only to the current hand-microrobot distance.

For robot training, the reward is written as the sum of four intuitive terms: distance regulation, action smoothness, workspace-boundary safety, and catch avoidance. Let d_t be the hand-microrobot distance, a_t be the robot action, and p_R,t be the microrobot position. The robot reward optimized by PPO is summarized as

rt = rdist(dt) + rsmooth(at) + rbound(pR,t) + rcatch(dt)    (1)

Equation (1) groups the robot reward into four components. The distance component r_dist(d_t) encourages the hand-microrobot separation to remain inside the ZPD band and penalizes deviations on either side. The smoothness component r_smooth(a_t) discourages abrupt commanded motion. The boundary component r_bound(p_R,t) penalizes microrobot positions outside the valid workspace. The catch component r_catch(d_t) penalizes hand-microrobot contact. Episodes terminate after catch, workspace exit, or the maximum horizon.

# III. METHODOLOGY

The robot must therefore regulate difficulty while hand behavior changes in both strategy and motor execution. Our approach treats this variability as a simulation problem, an observation problem, and a training-distribution problem. Cognitive-motor decoupled domain randomization broadens the simulated hand-behavior distribution by separating movement intent from motor-execution constraints. A dual-stream encoder makes hand-motion history available to the policy, while an auxiliary future-dynamics head adds a direct learning signal for short-horizon motion prediction. Iterative league training exposes the robot to a growing pool of hand policies so that the learned controller does not depend on one fixed virtual user.

## A. Cognitive-Motor Decoupled Domain Randomization

Hand behavior varies in both intended pursuit strategy and execution accuracy. The proposed domain-randomization strategy models these sources of variability separately by generating intended hand motion first and then applying motor-execution constraints, observation noise, and temporal delay. At the strategy level, a hand policy sampled from the hand-policy pool determines the intended displacement at each timestep. The pool contains a stochastic scripted pursuit controller (SPC) and trained RL hand policies. The SPC provides a transparent pursuit model, while learned hand policies can produce more anticipatory strategies such as interception.

For the stochastic scripted pursuit controller, let ρ_ep denote the episode-level stride length and q_t denote a planar unit direction. The intended displacement is

utSPC = ρepqt    (2)

The stride ρ_ep is sampled once at episode reset and then held fixed. The direction q_t follows an ε-greedy pursuit rule. With probability 1 - ε, q_t points from the current hand position toward the microrobot; with probability ε, q_t is sampled uniformly from all planar unit directions. In the experiments, ρ_ep is sampled uniformly between 0.45 and 0.70, and ε is fixed at 0.05. Learned hand policies output the intended displacement directly, and this displacement is passed through the same motor-execution model.

The raw desired displacement is first passed through a first-order low-pass filter to model motion inertia, as shown in (3).

xt = αxt−1 + (1−α)ut    (3)

where u_t denotes the raw desired displacement, x_t denotes the filtered displacement, and alpha in (0,1) is the inertia coefficient. Acceleration clipping is then applied componentwise to limit abrupt changes in motion, as shown in (4).

Δxt = clip(xt−xt−1, −amax, amax)    (4)

The delayed displacement is then applied to update the simulated hand position,

pH,t+1 = pH,t + Δxt−δ + ηt    (5)

where delta is sampled from the delay range and eta_t is zero-mean Gaussian position noise with covariance sigma^2 I. The delay buffer advances at 8 Hz in the motor-execution layer, so the 0--3 frame range in Table I corresponds to 0--375 ms. Table I lists the values and sampled ranges used for these constraints. Through this decoupled design, the robot policy is exposed to plausible hand-motion variations without tying training to a single simulated hand model.

TABLE I: MOTOR RANDOMIZATION PARAMETERS

| Parameter | Symbol | Range |
| --- | --- | --- |
| Motion inertia | alpha | 0.7 |
| Maximum acceleration | a_max | 0.15 |
| Gaussian observation noise | sigma | Uniform distribution over 0.01-0.08 |
| Neural delay | delta | 0-3 frames (0-375 ms) |

Uniform distribution denotes sampling uniformly over the stated interval.

## B. Dual-Stream Encoder with Future Dynamics Head

The proposed domain randomization scheme exposes the robot to diverse interaction behaviors during training. However, the robot does not observe the underlying behavior model directly. It must infer behavior patterns from the current interaction state and recent hand motion. The current hand position alone cannot distinguish deliberate pursuit from sensing noise, motor delay, or slow response. We therefore encode the observation with two streams, one for instantaneous geometric context and one for recent hand-motion history.

At each timestep, the 44-dimensional observation vector is divided into a 12-dimensional scalar vector and a 32-dimensional temporal buffer, as shown in (6).

ot = [st; ht−T:t]    (6)

The scalar component represents the instantaneous geometry of the interaction and is defined in (7).

st = [pR; pH; d(R,H); bN, bS, bE, bW; stride; at−1]    (7)

Here, p_R and p_H denote the two-dimensional positions of the microrobot and the hand. d(R,H) is their Euclidean distance. b_N, b_S, b_E, and b_W denote signed distances to the workspace boundaries. The variables stride and a_{t-1} represent the current movement step size and the previous robot action, respectively.

The temporal buffer h_{t-T:t} contains the T = 16 most recent relative hand displacement vectors, forming a 32-dimensional sequence. This history allows the encoder to estimate motion direction, response delay, and oscillatory behavior that are unavailable from the current position alone. Relative displacements make the history representation translation-invariant, so the recurrent stream focuses on how the hand is moving rather than where the interaction occurs in the workspace.

The scalar vector and temporal buffer are encoded in separate streams, as shown in Fig. 2. A multi-layer perceptron (MLP) extracts geometric information from the scalar state, while a gated recurrent unit (GRU) processes the displacement sequence. The two stream outputs are concatenated and passed through a fusion MLP to produce the shared representation used by the policy and value heads.

Fig. 2. Dual-stream policy architecture with auxiliary future dynamics. The spatial branch encodes the geometric state, the temporal branch encodes recent interaction history with a recurrent module, and the fused representation supports both the policy-value heads and the auxiliary future-motion prediction head.

The ZPD-based reward provides the main control signal, but it only supervises the encoder indirectly through policy optimization. To provide a denser learning signal for motion representation, we attach a future-dynamics head to the fused representation. The head predicts the relative hand displacement over an eight-step horizon. Because future displacements are available from the rollout, this auxiliary task is self-supervised and requires no additional annotation.

The trajectory prediction loss is given in (8).

Ltraj = E[‖D̂t+1:t+H−Dt+1:t+H‖₂2],  H=8    (8)

where D_hat_{t+1:t+H} is the predicted future displacement sequence and D_{t+1:t+H} is the observed future displacement sequence. The full training objective combines the PPO loss with the auxiliary trajectory-prediction loss in (9).

Ltotal = LPPO + λtrajLtraj    (9)

where lambda_traj controls the auxiliary loss weight. Because the future-dynamics head shares the encoder with the policy, its gradients also update the spatial and temporal streams. These auxiliary gradients encourage the encoder to capture motion inertia, delay, and interaction dynamics, helping the policy anticipate near-future hand motion from recent history.

## C. League Training

Even with a well-designed motion representation, achieving stable and adaptive difficulty regulation remains challenging when the training distribution lacks diversity in hand behaviors. A robot trained against a single hand policy may over-specialize to that policy and fail when the pursuit strategy changes. At the same time, training the robot and hand policies simultaneously creates a non-stationary learning problem. The actions of both agents jointly determine the next environment state, so each agent’s transition dynamics and resulting returns are directly influenced by the current policy of the other agent. When both policies are updated together, each update changes the environment faced by the other policy, violating the stationarity assumption of PPO. In practice, the interaction can enter a cycle in which the robot exploits the current hand policy, the hand adapts, and the robot loses strategies that were useful against earlier hands.

We address these issues by separating the two sides of training and by retaining older hand behaviors in a shared pool. During robot training, the hand policies are fixed and treated as part of the environment. During hand training, the robot policy is fixed. The robot is then trained against a hand-policy pool P that contains the SPC and learned hand policies from previous stages, exposing it to direct pursuit as well as more strategic interception patterns. The pool is used to broaden the hand-behavior distribution, not to make the task purely adversarial; the robot must maintain the ZPD interaction rather than simply maximize escape.

Algorithm 1 summarizes the iterative procedure. Each iteration first trains the robot against hands sampled from the fixed pool. It then trains a new hand policy against the frozen robot, initializes that hand from the previous learned hand when available, and adds the resulting policy to the pool for the next robot-training phase.

Algorithm 1: Iterative League Training

Input: initial robot policy π_R^(0), scripted pursuit controller π_H^SPC, number of league iterations N.

Initialize: hand-policy pool P ← {π_H^SPC}.

1. for n = 1, 2, ..., N do

2. Train robot policy π_R^(n) with PPO against hand policies sampled from P.

3. Initialize hand policy π_H^(n) from π_H^(n-1) when available.

4. Train π_H^(n) against the frozen robot policy π_R^(n).

5. Update hand-policy pool P ← P ∪ {π_H^(n)}.

6. end for

Uniform sampling from the learned-hand pool is inefficient because different learned hands challenge the current robot to different degrees. We therefore use recent episode length as a simple competitiveness signal. A shorter episode usually means that the hand quickly catches the microrobot or forces early termination, so that hand should be sampled more often during the next robot-training phase. For learned hand i, let l_bar_i be its recent mean episode length. The learned-hand sampling score is inversely related to this length and is smoothed with a uniform mixture:

si = 1ℓ̄i + εℓ,  p̃i = (1−μ)si∑sj + μm    (10)

Here, m is the number of learned hand policies, ε_l is a small stabilizing constant, and μ controls the uniform exploration mass. The SPC remains in the pool with a fixed sampling probability, while the learned-hand portion follows the distribution in (10). This keeps the scripted pursuit behavior available and concentrates the remaining robot-training episodes on learned hands that still expose weaknesses in the current robot policy.

# IV. EXPERIMENTS AND RESULTS

The evaluation combines simulation experiments with a physical deployment test. The simulation experiments measure robustness across hand behaviors and isolate the contribution of temporal hand-history encoding. The physical deployment test evaluates whether the learned policy can run inside the real-time perception-control stack on the UR10 platform and whether the realized motion responds coherently to measured hand movement.

## A. Simulation Protocol and Metrics

The simulation study separates two sources of performance gain in the proposed framework. The league analysis tests whether expanding the hand-policy pool improves robustness across pursuit behaviors. The ablation analysis tests whether temporal hand-motion encoding and the auxiliary dynamics objective improve the learned robot representation. Both analyses use the pursuit-evasion environment described in Section II, with the robot controlling the microrobot and the hand motion generated by the behavior models described in Section III.

All simulation evaluations used the same planar workspace and ZPD definition. A timestep was counted as in-zone when the hand-microrobot distance fell within the target ZPD interval. The simulator and the physical workspace share the same calibrated coordinate convention, so one workspace unit corresponds to 1 cm after calibration and can be compared directly with physical rollout distances.

Time-In-Zone (TIZ) is the primary metric. It measures the fraction of the full episode horizon for which the hand-microrobot distance remains inside the ZPD interval. Episode length is reported in simulation steps and measures how long the interaction remains active before catch, workspace exit, or the maximum horizon. For the league analysis, cross-hand robustness is summarized by mean TIZ across learned hand policies and by worst-hand TIZ. For the network ablation, TIZ is complemented by workspace coverage and mean jerk, which respectively measure task coverage and motion smoothness in the final policy evaluation.

The training-protocol comparison uses an SPC-only robot trained against the stochastic scripted pursuit controller, a single-hand robot trained against one learned hand policy, and a league-trained robot trained through iterative hand-pool expansion. These policies are evaluated through cross-generation learned-hand tests and through the scripted, learned, and manual hand interfaces used in Table II. The representation comparison keeps the training setting fixed while changing the policy encoder. The MLP baseline uses instantaneous geometry, the GRU policy adds recent relative hand displacement, and the auxiliary GRU policy adds the future-dynamics prediction objective.

## B. League Training and Cross-Hand Robustness

The league evaluation asks whether an expanding hand-policy pool improves robustness across pursuit behaviors, rather than only improving performance against the most recent hand policy. After each robot generation is trained, it is evaluated against all learned hand generations in the pool, separating generalization across hand behaviors from progress against a single training opponent. In Fig. 3(a), each row fixes one robot generation and each column fixes one learned hand generation. Early robot generations perform well only for limited parts of the hand pool, whereas later generations maintain stronger TIZ over a broader set of learned hands, suggesting reduced specialization. Fig. 3(b) summarizes the same matrix through mean and worst-hand TIZ. The mean curve reflects typical performance, and the worst-hand curve tracks the most difficult learned hand for each robot generation; both improve from early to final generations despite local fluctuations.

Fig. 3. Cross-hand validation of league-trained robot policies under the ZPD 3.5-5.5 simulation setting. (a) TIZ matrix obtained by evaluating each robot generation against each learned hand generation. Rows correspond to robot generations, and columns correspond to learned hand generations. (b) Mean and worst-hand TIZ across learned hand test policies for each robot generation.

Table II evaluates whether this robustness extends beyond the learned-hand pool used in Fig. 3. The final SPC-only, single-hand, and league-trained robot policies are tested against the stochastic scripted pursuit controller, a learned hand policy, and manual mouse pursuit. Manual mouse pursuit introduces human reaction time and voluntary pursuit strategy, so it is treated as a human-in-the-loop stress test rather than another automated hand policy. Higher TIZ and longer episode length indicate more sustained difficulty regulation.

TABLE II: POLICY COMPARISON ACROSS THREE HAND TEST SETS

| Robot policy | Test set | TIZ | Length |
| --- | --- | --- | --- |
| SPC-only | SPC | 0.31 | 52 |
| SPC-only | Learned hand | 0.19 | 31 |
| SPC-only | Manual mouse | 0.14 | 33 |
| Single-hand | SPC | 0.11 | 25 |
| Single-hand | Learned hand | 0.49 | 70 |
| Single-hand | Manual mouse | 0.19 | 40 |
| League | SPC | 0.38 | 67 |
| League | Learned hand | 0.48 | 64 |
| League | Manual mouse | 0.49 | 74 |

Each value is reported as a mean over 50 evaluation episodes for each test condition. TIZ denotes the mean fraction of the full episode horizon spent in the ZPD, and Length denotes mean episode length in simulation steps. Standard deviations are omitted to keep the policy comparison compact.

The two non-league baselines show complementary forms of specialization. The SPC-only robot remains most suitable for the scripted pursuit controller but degrades under learned and manual pursuit. The single-hand robot performs well against the learned hand policy but loses robustness when the pursuit behavior changes to scripted or manual control. The league-trained robot gives the most balanced profile across all three test sets, preserving learned-hand performance while improving the scripted and manual-pursuit cases. This balance is the desired behavior for adaptive difficulty regulation, because hand behavior can change across controllers, sessions, and voluntary strategies.

## C. Network Ablation and Auxiliary Dynamics Analysis

The league results evaluate the diversity of hand behaviors used during training, while the network ablation tests whether the robot policy can use temporal hand-motion information once that diversity is present. The MLP baseline uses the instantaneous geometric state only, the GRU policy adds the 16-frame relative-displacement history, and the auxiliary GRU further adds the future-dynamics prediction head described in Section III.B. The training curves in Fig. 4(a) and Fig. 4(b) show that temporal history improves optimization and interaction persistence. The GRU policies reach higher episode reward and longer episode length than the MLP baseline, indicating that current geometry alone is insufficient for stable difficulty regulation when hand motion changes over time. The auxiliary GRU gives the strongest training curve, suggesting that the prediction loss helps the recurrent stream form a more useful motion representation.

The auxiliary prediction examples in Fig. 4(c) provide a qualitative check on this representation: the predicted short-horizon displacement generally follows the direction and curvature of the observed future hand motion rather than only reconstructing the current position. The final normalized metrics in Fig. 4(d) summarize the control effect of these representation changes. Recurrent history improves in-zone behavior and workspace coverage relative to the MLP baseline, and adding the auxiliary prediction objective further improves these task-quality metrics while reducing mean jerk. Together, the ablation indicates that temporal hand history and auxiliary dynamics prediction contribute to smoother and more reliable difficulty regulation.

Fig. 4. Network ablation for temporal encoding and auxiliary dynamics prediction. (a) Smoothed episode reward during training for the MLP, GRU, and auxiliary GRU policies. (b) Smoothed episode length during training for the same policies. (c) Representative auxiliary-head predictions of short-horizon hand displacement compared with observed future motion. (d) Final task-quality metrics normalized to the MLP baseline; higher values are better for in-zone time and workspace coverage, whereas lower values are better for mean jerk.

## D. Real-Time Physical Deployment

The simulation-trained policy was deployed on the UR10 microrobot platform to evaluate zero-shot execution with real hand input. In each rollout, a healthy pilot pursued the passive microrobot on the acrylic workspace while the UR10 moved the magnetic actuator beneath the surface. The policy was used without physical fine-tuning, and the physical dataset contains nine zero-shot rollouts from one pilot session. The camera and UR10 command loops operated in real time, and policy inference was fast enough to support online closed-loop interaction.

The deployment stack preserved the simulation observation format. Policy inputs were constructed from calibrated camera measurements of the hand and microrobot, workspace-boundary features, the previous executed robot command, and recent relative hand-displacement history, matching the categories in (6) and (7). UR10 telemetry and end-effector pose were not provided to the policy; RTDE pose readback was used only for Cartesian command execution and monitoring over Ethernet. Perception and calibration were handled by the camera-based tracking pipeline, which combined YOLO microrobot detection, MediaPipe-based hand tracking, and homography-based image-to-workspace calibration. During short hand-tracking gaps, the controller used dead-reckoning from the last measured hand velocity. Safety constraints included workspace clipping, command-step limiting, automatic stop-on-catch, UR10 servoStop on shutdown or interruption, and the standard hardware emergency stop available during testing.

Fig. 5 presents one representative zero-shot physical rollout. The image sequence in Fig. 5(a) and the dynamic traces in Fig. 5(b) come from the same trial, so they describe a single time-aligned interaction rather than separate examples. The selected frames cover the initial approach, steady pursuit, faster and slower hand movements, and the end of the rollout, showing that camera perception, hand pursuit, microrobot motion, and UR10 actuation operated as one closed loop. The distance trace remains near the target ZPD band for a substantial portion of the trial and returns toward this range after deviations, indicating that the controller neither simply moves the microrobot away from the hand nor keeps it static until catch. The hand- and robot-speed profiles support the same interpretation: changes in measured hand speed are reflected in the distance trace and in the robot-speed response. The deployment therefore evaluates the complete camera-policy-robot loop, rather than offline policy inference alone, and shows that the simulation-trained controller can sustain an online hand-microrobot interaction on the UR10 platform.

Fig. 5. Zero-shot physical deployment on the UR10 microrobot platform. (a) Key deployment frames from a representative rollout among nine physical trials. (b) Smoothed hand-microrobot distance, target ZPD band, and measured hand and robot speeds from the same rollout.